# CHAPTER 4

PRELIMINARY WORKS: PROTOTYPE DESIGN, INTEGRATION, AND VALIDATION FRAMEWORK

INTELLI-GATE: An IoT-Based Smart Gate System with Temperature Screening, Headcount Monitoring, and Metal Detection

Chapter status: Design-stage technical revision. Engineering dimensions and conceptual components below originate from the preliminary manuscript and accompanying gate drawing. Unobserved implementations, test outcomes, acceptance criteria, and design refinements are explicitly identified as proposed or requiring verification. This chapter does not report fabricated experimental findings.

# 4.1 Chapter Overview

This chapter documents the preliminary design and prototyping framework of Intelli-Gate, a campus entrance monitoring prototype conceived to integrate non-contact temperature screening, bidirectional passage counting, zone-based inductive metal detection, and local/centralized reporting. Unlike a conventional presentation of product photographs, the discussion establishes traceability among the research objectives, physical assembly, electrical interfaces, software services, information flows, data structures, and validation evidence that should be collected in controlled trials. The primary present outputs are engineering designs, hardware selections, and functional specifications rather than verified system performance.

# 4.2 Design Basis, Requirements Traceability, and Prototype Status

The baseline described in Chapters 1–3 comprises a Raspberry Pi 4 Model B, MLX90614 temperature sensing, infrared break-beam counting, inductive sensing coils, buzzer, LCD, Python/Flask, SQLite, Flask-SocketIO, and a LAN-connected monitoring terminal. An additional ESP32-based coil excitation circuit is described in the existing Chapter 4. Its relationship to the Raspberry Pi must be specified as a subordinate sensing controller rather than assumed to be interchangeable with the main processor.

| Research objective | Physical/software subsystem | Observable verification evidence |
| --- | --- | --- |
| Temperature and metal screening | MLX90614; coil sensing; zone mapping; LCD | Reference thermometer comparisons; per-zone detection trials; displayed-alert consistency |
| Entry/exit headcount | Ordered pair of break-beam channels; event logic | Manual ground truth versus ENTRY/EXIT logged counts |
| Real-time centralized monitoring | Raspberry Pi + Flask/SocketIO + SQLite + LAN | Timestamped database records, screen captures, delivery latency |
| Functionality, accuracy, reliability and usability | Integrated prototype and research instruments | Trial matrix, repeated measures, failure logs and participant assessment |

Design reconciliation required. Objective 1 on manuscript page 5 refers to six metal-detection zones; scope, definition of terms, hardware bill, and data dictionary specify four: UPPER, MIDDLE, LOWER, BAG. The current gate elevation visually depicts three vertically stacked detection regions on each side. These are potentially six physical coil panels but not necessarily six independent logical classifications. This revision adopts four logical classes as the documented baseline, with the bag zone treated as a dedicated physical sensing requirement that has not been evidenced by the existing mechanical drawing. The final manuscript must consistently adopt either four or six zones and revise all objectives, sensor maps, software rules, and evaluation forms accordingly.

# 4.3 Mechanical Prototype and Spatial Layout

## 4.3.1 Walk-through portal geometry

The submitted prototype drawing specifies an overall height of approximately 2,200 mm, width of 900 mm and depth of 400 mm, with a passage opening approximately 700 mm wide and 2,000 mm high. The gate adopts paired lateral columns joined by a top crossmember that houses the control/display assembly. These dimensions describe the preliminary concept, not validated fabrication tolerances or structural load capacity. The final fabrication drawing should include a dimension datum, tolerances, conduit clearances, pedestrian accessibility check, and anti-tip/stability calculation.

Figure 4.1. Submitted walk-through gate engineering concept: elevations, views, exploded assembly and component arrangement (source: user-provided preliminary drawing).

| Geometry or material | Preliminary document value | Verification/design implication |
| --- | --- | --- |
| Overall height | 2,200 mm | Confirm measured assembled dimension and overhead clearance |
| Overall width | 900 mm | Account for actual installed footprint and lateral mounting |
| Overall depth | 400 mm | Determine base stability and enclosure access |
| Nominal walking clear width | 700 mm | Verify safety and accessibility against site requirements |
| Nominal clear height | 2,000 mm | Confirm structural tolerances |
| Structural core | 4-inch PVC pipes (document description) | Material suitability, bending/stability and fastening to be tested |
| Coil housing | FRP/ABS (document description) | Ensure nonmetallic housing and maintainable coil mounting |
| Base / finish | Cement-filled base; rubberized panels; black/chrome finish | Verify mass, anchoring, cable strain relief and trip hazards |

## 4.3.2 Sensor integration map

The vertical sections provide candidate locations for body-height coil assemblies. However, each panel must be associated with a named coil channel and a documented detection envelope. A bag zone cannot be assumed to exist merely because six physical side panels appear in the drawing. Placement of the bag coil, thermal reader, two ordered passage beams, and service openings should be separately shown in a revised instrumentation layout. The location of a temperature reading point should be defined so subject-to-sensor geometry remains repeatable.

| Proposed location | Functional device | Documentation still required |
| --- | --- | --- |
| Top/control crossmember | LCD, status indicators, optional buzzer | Display readability, enclosure dimensions, wiring |
| Left/right side columns | Upper/middle/lower metal-sensing assemblies | Channel assignments, shielding, false-alarm study |
| Dedicated bag inspection region | Bag coil, if within final scope | Mechanical mounting and independent test coverage |
| Approach side at defined height | MLX90614 non-contact thermal sensor | Standoff, field of view, environmental effects |
| Opposite ends of passage line | IR break-beam A and B | Sequence spacing and alignment |
| Service enclosure | Raspberry Pi / controller / supply / network | Service isolation, ventilation and cable labels |

# 4.4 Electrical and Embedded Hardware Prototyping

## 4.4.1 Main controller and data-acquisition partition

The Raspberry Pi 4 is identified in Chapter 3 as the primary processing unit. The existing Chapter 4 circuit introduces an ESP32 GPIO25 output, gate driver, IRLZ44N MOSFET, 24-V inductive load, 100-Ω gate resistor, 10-kΩ pulldown and a flyback diode. These components describe a possible coil excitation stage, but the circuit does not by itself demonstrate a complete metal detector: a receive/sense stage, signal acquisition, baseline compensation, interference controls, detection decision logic and safe power-domain boundaries are also needed. If an ESP32 is retained, it should act as a defined acquisition/excitation coprocessor that transmits calibrated event or signal data to the Pi using a documented interface. Electrical suitability and safety must be confirmed before energizing the circuit.

Engineering safety note: the 24-V coil power domain and the Raspberry Pi 3.3-V GPIO domain must be isolated or interfaced by appropriate level-shifting and driver circuitry. MOSFET voltage/current/transient ratings, coil inductance and stored energy, flyback path, heating, power supply current rating, fusing and grounding must be measured and verified. The schematic narrative alone does not establish that the sensing circuit is operational.

| Subsystem | Reference component | Signal / control pathway | Expected output |
| --- | --- | --- | --- |
| Central processor | Raspberry Pi 4 | GPIO, I²C, LAN | Unified gate state / monitoring services |
| Thermal | MLX90614 | I²C → Pi | Temperature estimate and timestamp |
| Passage | Two break-beam channels (recommended arrangement) | Digital transitions → Pi | Ordered ENTRY/EXIT event |
| Metal excitation/acquisition | Inductive coil + driver; ESP32 proposed in circuit | Conditioned acquisition → Pi | Zone ID + signal measurement / detection flag |
| Local annunciation | LCD + active buzzer | I²C / GPIO driver | Screen status and audible pattern |
| Networking / persistence | Ethernet/Wi-Fi / SQLite | Flask + SocketIO | Record storage and authorized dashboard |

## 4.4.2 Functional prototype controls

Electrical interface test: inspect voltage rails, continuity, polarity, protective components and grounding before integration.

Sensor test: record the raw value, sampling period, signal stability, saturation and noise characteristics of each channel.

Environmental test: test changes in ambient temperature, electromagnetic interference, and pedestrian proximity.

Recovery test: document startup sequence, unplug/replug events, controlled network loss, process restart and power restoration.

Safety design: make any barrier actuator subordinate to explicit human safety controls and a manual release; actuation is not established by the present gate drawing.

# 4.5 Logical System Architecture and Communication Layout

A layered edge-monitoring architecture is proposed for traceable integration. Sensor channels acquire physical signals; an embedded collection layer validates and timestamps values; the Raspberry Pi processes screening events and applies configuration rules; SQLite persists the event logs; and Flask-SocketIO publishes authorized views over the local campus network. Local display and alarms should remain usable under LAN failure where feasible. This is a recommended integration topology consistent with the listed technologies, not evidence of completed installation.

Figure 4.2. Proposed full system architecture and interfaces (design representation, not verified deployment).

# 4.6 Data Flow Design and Operational Sequences

The context diagram defines interactions with the monitored entrance, authorized personnel, and output devices. The Level 1 DFD separates sensor acquisition, normalization, classification and passage counting, alert decisions, storage, and presentation. The Level 2 decompositions provide a specification for validation of data inputs, decision rules, event storage and acknowledgment. These diagrams should be reconciled with the final physical interfaces and must maintain balanced flows between parent and child process levels.

Figure 4.3. System context DFD (logical system boundary).

Figure 4.4. Level 1 DFD (principal processing and persistence functions).

Figure 4.5. Level 2 sensor screening decomposition (proposed).

Figure 4.6. Level 2 alerts/dashboard decomposition (proposed).

## 4.6.1 Operational event sequence

1. Presence or sensor activation initiates an event window; acquisition timestamps synchronized input samples.

2. The thermal channel acquires a reading and attaches validity metadata (e.g., sensor in-range, stable measurement).

3. The metal sensing subsystem evaluates the configured zones using documented baseline and detection thresholds.

4. Ordered beam interruptions are evaluated to distinguish ENTRY, EXIT, incomplete passage or ambiguous events.

5. The software stores the underlying source observations and generates notifications according to approved decision rules.

6. The local LCD/buzzer and LAN dashboard display event outcomes; an authorized person verifies suspected threats and acknowledges alerts.

# 4.7 Prototype Data Model, Logging, and Integrity

The preliminary Chapter 3 data dictionary identifies users, headcount logs, temperature logs, metal-detection logs, alerts and settings. Their proposed fields can support an initial SQLite design. However, its polymorphic alert source_id is described as a foreign key to either temperature_logs or metal_detection_logs; a single conventional SQLite foreign key cannot enforce a reference to two target tables. The implementation should instead use explicit nullable temperature_id and detection_id foreign keys with a mutual-exclusivity rule, or a normalized parent event table. Event correlation IDs are recommended to associate synchronous readings without identifying students.

| Dataset | Minimum essential recorded data | Quality/integrity control |
| --- | --- | --- |
| temperature_logs | temp_id, value, threshold, result, reading_time | Reading validity; units; timestamp; device identifier |
| headcount_logs | count_id, ENTRY/EXIT, sensor_id, event_time, occupancy | Atomic count update; ambiguous-passage flag |
| metal_detection_logs | detection_id, zone, signal, flag, detection_time | Known zone code; units / threshold provenance |
| alerts | alert_id, type, source, message, time, status, actor | Referential integrity; immutable creation timestamp |
| users / settings | hashed credential / role; named parameter and revision | Least privilege; auditable configuration changes |

Recommended system state constraints: occupancy should not be negative; historical records should not be overwritten by later counter resets; sensor failures should be distinguishable from normal readings; duplicate rapid transitions should be suppressed only by validated event logic, not by arbitrary removal of data. No personal identity capture is required for the core anonymous headcount objective.

# 4.8 Prototype Verification Plan and Data Collection Instruments

The preliminary manuscript describes unit, integration, functional, accuracy, reliability and response-time tests, but Chapter 4 does not presently supply measured trial data. Accordingly, the tables in this subsection are prospective data-collection formats and MUST NOT be labeled results until verified observations are entered. Calibration procedure, comparison standard, sample size and acceptance limits should be preregistered and justified before testing.

## 4.8.1 Test matrix aligned with research objectives

| Test ID / function | Manipulated condition | Reference truth / evidence | Primary metric |
| --- | --- | --- | --- |
| T01 thermal | Reference temperatures, varied standoff | Calibrated reference thermometer and controlled setup | MAE, bias and reading availability |
| T02 zone metal | Metal/no-metal across each designated zone | Labeled known trials with coil configurations | Sensitivity, specificity; confusion matrix by zone |
| T03 direction | Single and two-person passage sequences | Manual tally/video-free observer log | Entry/exit accuracy; ambiguous event rate |
| T04 end-to-end | Repeated integrated entries | Sensor timestamp, DB timestamp, UI arrival time | p50/p95 dashboard latency |
| T05 reliability | Long-run operation; sensor/network interruptions | System/uptime logs and recovery observation | Successful event ratio; recovery time |
| T06 usability | Guard/admin guided tasks | Task completion, timing, survey and interview | Completion %, error count, usability ratings |

## 4.8.2 Raw trial log templates

| Trial | Reference °C | Sensor °C | Abs. error °C | Distance / conditions | Validity |
| --- | --- | --- | --- | --- | --- |
| T-001 | — | — | — | — | Not tested |
| T-002 | — | — | — | — | Not tested |
| T-003 | — | — | — | — | Not tested |

| Trial | Expected zone | Test object / placement | Detected zone(s) | Signal / threshold | Correct? |
| --- | --- | --- | --- | --- | --- |
| M-001 | UPPER | — | — | — | Not tested |
| M-002 | MIDDLE | — | — | — | Not tested |
| M-003 | LOWER | — | — | — | Not tested |
| M-004 | BAG | — | — | — | Not tested |

| Trial | Actual entries | Actual exits | Logged entries | Logged exits | Ambiguous events |
| --- | --- | --- | --- | --- | --- |
| H-001 | — | — | — | — | Not tested |
| H-002 | — | — | — | — | Not tested |

| Trial | Detection time | Log commit time | Dashboard render time | Alert result | Remarks |
| --- | --- | --- | --- | --- | --- |
| L-001 | — | — | — | Not tested | — |
| L-002 | — | — | — | Not tested | — |

## 4.8.3 Statistical measures and interpretation

Thermal error: eᵢ = sensorᵢ − referenceᵢ; bias = (Σeᵢ)/n; MAE = (Σ|eᵢ|)/n; repeatability may be expressed using SD under held-constant conditions. Metal binary performance: sensitivity = TP/(TP+FN), specificity = TN/(TN+FP), with separate zone confusion matrices and clear definitions of positive metal and correct localization. Counting: event-level accuracy = correct classified passages / valid labeled passages, reported alongside net occupancy error = estimated occupancy − ground-truth occupancy. Latency: render time − detection time, reporting median and 95th percentile with synchronized clocks. For repeatability, report trials, uncertainty, missing events and confidence intervals where applicable. Percentages alone are insufficient without denominators.

A measured body-surface/forehead infrared temperature reading should not be interpreted automatically as core body temperature or diagnostic evidence of fever. Device installation and the screening cutoff must be tested with documented measurement conditions. The metal detector cannot reliably infer weapon type or dangerousness simply from a metal/no-metal signal. Any special alarm category requires a separate, validated classification rule; authorized security personnel must make the final assessment.

# 4.9 Software Quality and Stakeholder Evaluation

As described in Chapter 3, selected characteristics from ISO/IEC 25010 guide assessment. Objective tests should evaluate functional suitability and reliability, while structured participant ratings and task observations address interaction/usability. Because the current work is preliminary, evaluative claims should be made only after prototype trials and completed questionnaires. The questionnaires should use characteristic-specific indicators, and results should report participant number, recruitment criteria, scale anchors, item-level distributions, means where justified, and pertinent qualitative themes.

| Evaluation dimension | Observable operational indicator | Evidence source |
| --- | --- | --- |
| Functional suitability | All required outputs generated for defined scenarios | Test scripts and screenshots |
| Performance efficiency | Event processing and display latency | Instrumented timestamps |
| Reliability | Repeated correct operation and recovery behavior | Trial log / fault injection |
| Usability / interaction quality | Task success, comprehension and workload | Observed tasks, survey and interview |
| Data protection | Authorized access, minimized retention, auditability | Configuration inspection and policy review |

# 4.10 Prototype Limitations, Risk Controls, and Design Decisions

Zone discrepancy: four logical classes versus three vertical sections on each side versus a six-zone objective; freeze one tested architecture before claiming compliance.

Sensor independence: multiple coils may interfere electromagnetically; document sampling multiplexing or shielding and validate per-zone specificity.

Thermal validity: infrared surface readings vary with geometry and environment; control conditions and avoid diagnostic claims.

Counting limitations: simultaneous or reversing pedestrians produce ambiguous edge sequences; test and report rather than silently discard.

Power/network resilience: loss of LAN must not hide critical local alerts; define offline storage, retry behavior, and safe restart.

Privacy and security: collect anonymous operational events where possible; restrict dashboard access and define retention/secure deletion under institutional policy and applicable Philippine privacy obligations.

Physical safety: assess tip stability, electrical enclosure, sharp edges, trip points and barriers to emergency egress; no actuator should block evacuation.

# 4.11 Prototype Readiness and Evidence Gaps

| Deliverable / evidence | Current basis | Status to report |
| --- | --- | --- |
| CAD gate dimensions and exploded views | Submitted mechanical concept and description | Design presented; fabrication verification pending |
| Metal coil driver | ESP32–driver–MOSFET schematic narrative | Design described; full receive electronics and test evidence pending |
| Four-zone implementation | Chapter 1/3 and database schema | Inconsistent with six-zone wording / gate panels |
| Thermal/beam mounting locations | Component list only | Installation drawings pending |
| Raspberry Pi + Flask dashboard | Proposed software stack | Live deployed screenshots / measurements not supplied |
| Database schema | Chapter 3 data dictionary | Logical design documented; integrity testing pending |
| Accuracy, latency and reliability | Testing plans without raw measurements | Results cannot yet be reported |

# 4.12 Chapter Synthesis

The preliminary work establishes a credible multidisciplinary design direction: a portal enclosure capable of accommodating inductive metal-sensing elements and electronic monitoring hardware, an embedded processing concept centered on the Raspberry Pi, and a local network architecture designed to consolidate entrance-related events. However, the available documentary evidence is primarily conceptual and design-stage. No measured claims of detection accuracy, directional counting performance, temperature error, response latency, reliability or operational usability can be inferred from drawings alone. Completion of the research objectives therefore requires consistent zone definitions, verified sensor/circuit interfaces, engineering safety checks, instrumented test trials, and auditable data records. Once these measurements are collected, this chapter should be extended with empirical results and objective-by-objective analysis rather than replacing planned test tables with unsupported performance values.

# Documentation notes for the final manuscript

Figure numbering in this revision is provisional and should be updated with automatic Word captions once the complete manuscript is consolidated. The electrical schematic itself should be reproduced from the original source figure at adequate resolution, with component values checked against the fabricated circuit. The architecture/DFDs are supplementary proposed design figures and are not substitutes for evidence of actual integration. Dates and software versions should reflect the final prototype.

# 4.13 Consolidated Hardware Set-up and Pin Mapping

To strengthen the prototyping documentation, this revision adds two full-HD technical figures derived from the consolidated bill of materials (BOM), the walk-through gate concept, the ESP32 coil-driver prototype notes, and the Raspberry Pi-based architecture described in Chapter 3. These figures are intended to improve design traceability by showing how the physical gate, embedded controllers, sensor interfaces, power domains, and the browser-based dashboard are organized as a single prototype system.

The following figures should be interpreted as a research-grade reference implementation. They do not substitute for laboratory verification, continuity testing, power-budget measurement, signal-integrity checks, calibration, or final fabrication drawings. Nevertheless, they provide a defensible technical baseline for build planning, panel presentation, and objective-by-objective validation.

## 4.13.1 Full hardware set-up based on the consolidated BOM

Figure 4.8 presents the full hardware set-up of Intelli-Gate in integrated form. It consolidates the walk-through portal geometry, the physical placement of the body-level detection coils, the dedicated bag-zone concept, the MLX90614 thermal sensor, the paired break-beam headcount sensors, the top control panel, the Raspberry Pi 4 main controller, the ESP32-based metal-detection subcontroller, the power-supply arrangement, and the LAN-connected command-centre dashboard. The figure is especially useful in clarifying that the project is not merely a metal detector frame; rather, it is an IoT-enabled entrance-screening system whose components must function as a coordinated whole.

Figure 4.8. Full-HD consolidated hardware set-up of Intelli-Gate based on the manuscript BOM and prototype notes.

As shown in Figure 4.8, the walk-through frame provides the physical housing for the sensing and monitoring components. The upper, middle, and lower body channels are shown as distinct coil positions, while the bag zone is presented as a dedicated functional zone that must be finalized in the physical design if the study adopts the four-zone logical model. The top control panel serves as the local status interface, containing the LCD and local indicators, whereas the internal service enclosure houses the control electronics and wiring terminations. On the systems side, the Raspberry Pi performs the principal control, logging, and dashboard-serving functions, while the ESP32 is modeled as a subordinate metal-detection controller responsible for the coil-related channel management. The dashboard path through the router/switch and the command-centre computer makes explicit the intended real-time monitoring capability described in the earlier chapters.

## 4.13.2 Proposed pin-to-pin and interface connection map

Figure 4.9 translates the hardware architecture into a pin-to-pin and interface-level wiring map. This figure responds directly to the need for a clearer engineering explanation of how the ESP32, Raspberry Pi, peripheral sensors, and the dashboard are interconnected. In the proposed configuration, the Raspberry Pi remains the main controller and the LAN server host, while the ESP32 is assigned to the coil-driver and metal-detection subsystem. The figure also distinguishes logic/data lines, power paths, actuator outputs, and the serial/network bridge through color-coded links.

Figure 4.9. Reference pin-to-pin and interface connection map from the ESP32 to the Raspberry Pi and the LAN dashboard.

The proposed reference wiring maps the MLX90614 and the I²C LCD to the Raspberry Pi through GPIO2/SDA1 and GPIO3/SCL1, with power and ground drawn from the Pi header. The break-beam outputs are mapped to GPIO17 and GPIO27 to support ordered-sequence entry/exit logic, while GPIO22 is assigned to the buzzer or alarm driver. UART is used as the explicit bridge between the ESP32 and the Raspberry Pi, thereby providing a transparent integration point between the metal-detection subcontroller and the main monitoring controller. The ESP32 then distributes control to the coil-driver front-end through separate channel pins representing the logical upper, middle, lower, and bag-zone paths. Ethernet connects the Raspberry Pi to the router/switch, which in turn makes the monitoring interface available to the dashboard computer through a web browser.

Because the original manuscript does not yet publish a final fabricated wiring harness, the pin selections shown in Figure 4.9 should be treated as proposed reference allocations. Final implementation may revise specific GPIO assignments provided that the functional role, traceability, and software configuration are updated consistently in the manuscript, source code, and test documentation.

Figure 4.10. Full System Diagram.

## 4.13.3 Connection matrix summary

Table 4.10 summarizes the principal connections represented in the hardware and wiring figures. This matrix is useful for assembly planning, circuit review, and final validation because it identifies the expected functional interface of each subsystem. During prototype fabrication, this table should be updated to match the exact GPIO assignments, terminal labels, wire colors, and harness identifiers used in the physical build.

| Subsystem / device | Connected to | Interface type | Reference pins / terminals | Function in the prototype |
| --- | --- | --- | --- | --- |
| MLX90614 temperature sensor | Raspberry Pi 4 | I²C | Pin 1 (3V3), Pin 6 (GND), Pin 3 (SDA1), Pin 5 (SCL1) | Forehead/face-level thermal reading |
| I²C LCD 20×4 / 16×2 | Raspberry Pi 4 | I²C | Pin 2 (5V), Pin 6 (GND), Pin 3 (SDA1), Pin 5 (SCL1) | Local monitoring display |
| Break-beam sensor A | Raspberry Pi 4 | Digital input | GPIO17 / Pin 11 + 5V + GND | First ordered beam in entry/exit logic |
| Break-beam sensor B | Raspberry Pi 4 | Digital input | GPIO27 / Pin 13 + 5V + GND | Second ordered beam in entry/exit logic |
| Buzzer / alarm module | Raspberry Pi 4 | Digital output | GPIO22 / Pin 15 through driver transistor | Local alerting for alarms |
| Optional servo / gate motor | Raspberry Pi 4 | PWM / control output | PWM-capable GPIO (e.g., Pin 12 or Pin 32) + external 5V | Optional actuation only |
| ESP32 serial bridge | Raspberry Pi 4 ↔ ESP32 | UART | Pi Pin 8 (TXD0) ↔ ESP32 RX2; Pi Pin 10 (RXD0) ↔ ESP32 TX2 | Transfers metal-detection data / control |
| Metal channel CH1 | ESP32 | Digital output to driver | GPIO25 | Upper-body metal-detection path |
| Metal channel CH2 | ESP32 | Digital output to driver | GPIO26 | Middle-body metal-detection path |
| Metal channel CH3 | ESP32 | Digital output to driver | GPIO27 | Lower-body metal-detection path |
| Metal channel CH4 | ESP32 | Digital output to driver | GPIO14 | Bag-zone metal-detection path |
| Dashboard network | Raspberry Pi 4 → router/switch → dashboard PC | LAN / Ethernet | RJ45 Ethernet port | Flask-SocketIO monitoring interface |

## 4.13.4 Interface

Figure 4.11. Interface -Intelli-gate.