# INTELLI-GATE

# FINAL SYSTEM DATA DICTIONARY — SIX-ZONE IMPLEMENTATION

Research capstone | Implementation specification v1.0 | Proposed relational schema and data contract

Scope: Six metal-detection channels, temperature screening, entry/exit counting, ESP32-to-Raspberry Pi telemetry, SQLite persistence, alerts, settings, security accounts and LAN dashboard. This dictionary is a finalized design proposal, not proof of installed hardware or validated detection performance.

Each table includes field names, SQLite data types, primary and foreign keys, nullability, definitions, validation constraints, and example values.

# 1. Basis and reconciliation of the six-zone implementation

The source manuscript defines users, headcount logs, temperature logs, metal detection logs, alerts and system settings; its narrative ERD also describes passages and metal zones that were absent from the six published field tables. This design resolves the omission by implementing explicit PASSAGES and METAL_ZONES tables, plus device, communication and audit records required for a complete multi-controller system. Field names in the earlier dictionary are retained where technically sound, with passage identifiers, timestamps, foreign keys and state flags added to support referential integrity.

Zone resolution: the gate prototype has three detector panels on the left and three on the right, yielding six separately addressable physical positions. This version selects LEFT/RIGHT × UPPER/MIDDLE/LOWER, not the previous UPPER/MIDDLE/LOWER/BAG logical taxonomy. A bag-specific detection pathway is outside this exact six-zone layout and must not be claimed without further hardware and chapter revisions.

| zone_id | zone_code | side | level | physical meaning |
| --- | --- | --- | --- | --- |
| 1 | L_UPPER | LEFT | UPPER | Left upper coil panel |
| 2 | L_MIDDLE | LEFT | MIDDLE | Left middle coil panel |
| 3 | L_LOWER | LEFT | LOWER | Left lower coil panel |
| 4 | R_UPPER | RIGHT | UPPER | Right upper coil panel |
| 5 | R_MIDDLE | RIGHT | MIDDLE | Right middle coil panel |
| 6 | R_LOWER | RIGHT | LOWER | Right lower coil panel |

Core design principles: anonymous passage records (no biometric identity), immutable raw detection logs, normalized six-zone master data, clock-synchronized event records, configuration change histories, authentication with hashed passwords, and fail-state representation of sensor or network faults.

# 2. Relational data dictionary — SQLite implementation

## Table DD-01. users

Authorized dashboard user accounts. Passwords are salted and hashed using a maintained password-hashing library; plaintext credentials are never persisted.

| Field | SQLite type / key | Null? | Definition / validation | Example |
| --- | --- | --- | --- | --- |
| user_id | INTEGER PK AUTOINCREMENT | No | Stable account identifier | 1 |
| full_name | TEXT NOT NULL | No | 1–100 characters | Security Officer |
| role | TEXT CHECK | No | ADMIN, GUARD, VIEWER | GUARD |
| username | TEXT UNIQUE NOT NULL | No | 1–50 characters; case-insensitive unique indexing recommended | guard01 |
| password_hash | TEXT NOT NULL | No | Hashed password string | argon2id$… |
| is_active | INTEGER CHECK(0,1) | No | Logical activation state; default 1 | 1 |
| created_at | TEXT NOT NULL | No | ISO-8601 UTC timestamp | 2026-10-22T08:00:00Z |
| last_login | TEXT | Yes | Latest successful login UTC | 2026-10-22T09:00:00Z |

Integrity / processing rules: UNIQUE(username); authorized-role checks; never expose password_hash in API responses.

## Table DD-02. devices

Registered processing units and sensors participating in the embedded system.

| Field | SQLite type / key | Null? | Definition / validation | Example |
| --- | --- | --- | --- | --- |
| device_id | INTEGER PK | No | Unique device record | 1 |
| device_code | TEXT UNIQUE | No | Stable physical device label | ESP32-01 |
| device_type | TEXT CHECK | No | ESP32, RASPBERRY_PI, MLX90614, BREAK_BEAM, COIL, LCD, BUZZER | ESP32 |
| location | TEXT | Yes | Mounting position or service cabinet | Left service column |
| interface_type | TEXT | Yes | UART, GPIO, I2C, ETHERNET, WIFI | UART |
| firmware_version | TEXT | Yes | Version reported by device | 1.0.0 |
| is_enabled | INTEGER CHECK(0,1) | No | Administrative enable state | 1 |
| last_seen_at | TEXT | Yes | Most recent communication / heartbeat | 2026-10-22T09:10:00Z |

Integrity / processing rules: Device codes are stable even when a replacement hardware unit is commissioned.

## Table DD-03. metal_zones

Six-row master reference table; zone numbers are fixed identifiers corresponding to exact detector positions.

| Field | SQLite type / key | Null? | Definition / validation | Example |
| --- | --- | --- | --- | --- |
| zone_id | INTEGER PK CHECK 1..6 | No | 1–6 only | 1 |
| zone_code | TEXT UNIQUE NOT NULL | No | One of L_UPPER, L_MIDDLE, L_LOWER, R_UPPER, R_MIDDLE, R_LOWER | L_UPPER |
| side | TEXT CHECK | No | LEFT or RIGHT | LEFT |
| body_level | TEXT CHECK | No | UPPER, MIDDLE or LOWER | UPPER |
| coil_device_id | INTEGER FK devices | Yes | Coil registered for this detection position | 9 |
| threshold_value | REAL | Yes | Per-zone baseline/configured threshold (unit documented separately) | 0.65 |
| signal_unit | TEXT | No | Calibrated mV, ADC counts, or dimensionless normalized score | ADC_COUNT |
| is_enabled | INTEGER CHECK(0,1) | No | 1 if channel participates in active detection | 1 |

Integrity / processing rules: Initial rows exactly Z01–Z06; use UNIQUE(side, body_level). Avoid hardcoding qualitative severity by side or body level.

## Table DD-04. passages

Correlates multiple sensor readings to an observable transit through the frame.

| Field | SQLite type / key | Null? | Definition / validation | Example |
| --- | --- | --- | --- | --- |
| passage_id | INTEGER PK | No | Unique event-session identifier | 1001 |
| gate_id | TEXT NOT NULL | No | Gate installation label | GATE-01 |
| direction | TEXT CHECK | No | ENTRY, EXIT, UNKNOWN | ENTRY |
| started_at | TEXT NOT NULL | No | First relevant sensor timestamp (UTC) | 2026-10-22T09:20:01.000Z |
| completed_at | TEXT | Yes | Last event timestamp UTC | 2026-10-22T09:20:02.400Z |
| passage_status | TEXT CHECK | No | OPEN, COMPLETED, AMBIGUOUS, ABORTED | COMPLETED |
| counted | INTEGER CHECK(0,1) | No | Whether a headcount event was accepted | 1 |
| confidence_score | REAL CHECK 0..1 | Yes | Algorithm score if empirically defined | 0.93 |

Integrity / processing rules: UNKNOWN / AMBIGUOUS are essential for reversal or simultaneous passage; counted only when event rule confirms a direction.

## Table DD-05. headcount_logs

Append-only accepted directional passage counts; net occupancy derived atomically from these events.

| Field | SQLite type / key | Null? | Definition / validation | Example |
| --- | --- | --- | --- | --- |
| count_id | INTEGER PK | No | Unique count event | 2001 |
| passage_id | INTEGER FK passages UNIQUE | No | Reference to counted passage | 1001 |
| event_type | TEXT CHECK | No | ENTRY or EXIT | ENTRY |
| sensor_id | TEXT NOT NULL | No | Logical beam-pair identifier (not just one beam) | PC-PAIR-01 |
| event_time | TEXT NOT NULL | No | Time of count acceptance UTC | 2026-10-22T09:20:02Z |
| current_occupancy | INTEGER CHECK >=0 | No | Occupancy after accepted event | 24 |
| sequence_pattern | TEXT NOT NULL | No | A_B or B_A | A_B |

Integrity / processing rules: Count update and event insert must occur in one database transaction. EXITS at zero require reconciliation/exception handling, not silent negative occupancy.

## Table DD-06. temperature_logs

Non-contact temperature observations used for preliminary screening only, not medical diagnosis.

| Field | SQLite type / key | Null? | Definition / validation | Example |
| --- | --- | --- | --- | --- |
| temp_id | INTEGER PK | No | Unique reading ID | 3001 |
| passage_id | INTEGER FK passages | Yes | Associated transit if correlated | 1001 |
| device_id | INTEGER FK devices | No | Sensor producing measurement | 3 |
| temperature_value | REAL NOT NULL | No | Measured apparent °C; validate operating envelope | 36.6 |
| threshold_value | REAL NOT NULL | No | Configured threshold at capture | 37.5 |
| screening_result | TEXT CHECK | No | NORMAL, ELEVATED, INVALID | NORMAL |
| reading_quality | TEXT CHECK | No | VALID, OUT_OF_RANGE, SENSOR_ERROR, NOT_ALIGNED | VALID |
| reading_time | TEXT NOT NULL | No | UTC capture time | 2026-10-22T09:20:01Z |
| distance_mm | REAL | Yes | Measured/controlled standoff if available | 65 |

Integrity / processing rules: Do not conflate surface infrared readings with clinical core temperature; environmental/standoff conditions must be tested.

## Table DD-07. metal_detection_logs

One append-only per-channel detection observation or triggered metal event; multiple zone records may share one passage.

| Field | SQLite type / key | Null? | Definition / validation | Example |
| --- | --- | --- | --- | --- |
| detection_id | INTEGER PK | No | Unique observation identifier | 4001 |
| passage_id | INTEGER FK passages | Yes | Current correlated passage, if available | 1001 |
| zone_id | INTEGER FK metal_zones NOT NULL | No | Exact Z01–Z06 foreign key | 2 |
| esp32_device_id | INTEGER FK devices | No | Originating metal controller | 1 |
| signal_value | REAL NOT NULL | No | Measured signal with stated engineering unit | 142.50 |
| signal_unit | TEXT NOT NULL | No | ADC_COUNT, mV, NORMALIZED | ADC_COUNT |
| threshold_value | REAL NOT NULL | No | Threshold active for this reading | 120.00 |
| metal_detected | INTEGER CHECK(0,1) | No | Binary rule outcome | 1 |
| detection_time | TEXT NOT NULL | No | Source event time UTC | 2026-10-22T09:20:01Z |
| sample_seq | INTEGER NOT NULL | No | Monotonic ESP32 sequence within boot session | 308 |
| quality_flag | TEXT CHECK | No | VALID, SATURATED, NOISY, OFFLINE | VALID |

Integrity / processing rules: Do not store a generic “zone” text field in place of zone_id. Unique deduplication key can be (esp32_device_id, boot_id, sample_seq, zone_id); boot_id in telemetry frame.

## Table DD-08. alerts

Structured actionable warnings for metal, temperature, occupancy, sensor, or communication status.

| Field | SQLite type / key | Null? | Definition / validation | Example |
| --- | --- | --- | --- | --- |
| alert_id | INTEGER PK | No | Unique alert identifier | 5001 |
| alert_type | TEXT CHECK | No | METAL, TEMPERATURE, OCCUPANCY, SENSOR_FAULT, COMMUNICATION | METAL |
| severity | TEXT CHECK | No | INFO, WARNING, CRITICAL (configured policy) | WARNING |
| passage_id | INTEGER FK passages | Yes | Associated passage | 1001 |
| temperature_log_id | INTEGER FK temperature_logs | Yes | Populated for temperature alert | NULL |
| metal_detection_id | INTEGER FK metal_detection_logs | Yes | Populated for metal alert | 4001 |
| zone_id | INTEGER FK metal_zones | Yes | Applicable metal zone | 2 |
| alert_message | TEXT NOT NULL | No | Human-readable bounded message | Metal detected: left middle |
| alert_time | TEXT NOT NULL | No | UTC creation timestamp | 2026-10-22T09:20:01Z |
| alert_status | TEXT CHECK | No | ACTIVE, ACKNOWLEDGED, RESOLVED | ACTIVE |
| acknowledged_by | INTEGER FK users | Yes | Responsible account | 1 |
| acknowledged_at | TEXT | Yes | Time acknowledged | 2026-10-22T09:21:00Z |
| resolved_at | TEXT | Yes | Time manually/automatically resolved | NULL |

Integrity / processing rules: Separate explicit foreign keys replace source_id polymorphic pseudo-FK from the preliminary dictionary. Validate trigger association by alert_type.

## Table DD-09. system_settings

Current configurable system parameters and reference revision.

| Field | SQLite type / key | Null? | Definition / validation | Example |
| --- | --- | --- | --- | --- |
| setting_id | INTEGER PK | No | Settings record ID | 1 |
| setting_name | TEXT UNIQUE NOT NULL | No | Machine-readable setting identifier | temp_threshold_c |
| setting_value | TEXT NOT NULL | No | Serialized value parsed by declared type | 37.5 |
| value_type | TEXT CHECK | No | NUMBER, INTEGER, BOOLEAN, STRING, JSON | NUMBER |
| unit | TEXT | Yes | °C, ms, ADC_COUNT or NULL | °C |
| updated_by | INTEGER FK users | Yes | Account responsible for change | 1 |
| revision | INTEGER NOT NULL | No | Increment on update | 3 |
| updated_at | TEXT NOT NULL | No | UTC change timestamp | 2026-10-22T08:30:00Z |

Integrity / processing rules: Record settings history separately. Device-specific per-zone thresholds can reside in metal_zones with synchronized revision.

## Table DD-10. settings_audit_logs

Immutable record of administrative configuration changes for accountability and experimental reproducibility.

| Field | SQLite type / key | Null? | Definition / validation | Example |
| --- | --- | --- | --- | --- |
| audit_id | INTEGER PK | No | Change record ID | 6001 |
| setting_id | INTEGER FK system_settings | No | Setting changed | 1 |
| changed_by | INTEGER FK users | No | User who applied change | 1 |
| old_value | TEXT | Yes | Prior configuration | 37.8 |
| new_value | TEXT NOT NULL | No | Replacement configuration | 37.5 |
| reason | TEXT | Yes | Change justification | Calibration adjustment |
| changed_at | TEXT NOT NULL | No | UTC audit timestamp | 2026-10-22T08:30:00Z |

Integrity / processing rules: Append only; enforce policy prohibiting ordinary operator deletion.

## Table DD-11. device_heartbeats

Operational health records describing controller availability and transport state.

| Field | SQLite type / key | Null? | Definition / validation | Example |
| --- | --- | --- | --- | --- |
| heartbeat_id | INTEGER PK | No | Unique health observation | 7001 |
| device_id | INTEGER FK devices | No | Reporting board/device | 1 |
| observed_at | TEXT NOT NULL | No | UTC receive/capture timestamp | 2026-10-22T09:30:00Z |
| device_status | TEXT CHECK | No | ONLINE, DEGRADED, OFFLINE, ERROR | ONLINE |
| uptime_seconds | INTEGER CHECK>=0 | Yes | Source device uptime | 3700 |
| supply_voltage | REAL | Yes | Measured voltage, if instrumentation exists | 3.29 |
| details_json | TEXT | Yes | Optional diagnostic object | {"uart_errors":0} |

Integrity / processing rules: Heartbeat frequency and offline timeout are configuration controlled; measurements absent from actual hardware remain NULL.

## Table DD-12. controller_messages

Durable ESP32–Pi communications journal to support sequence integrity and link diagnostics.

| Field | SQLite type / key | Null? | Definition / validation | Example |
| --- | --- | --- | --- | --- |
| message_id | INTEGER PK | No | Pi-side message identifier | 8001 |
| source_device_id | INTEGER FK devices | No | Usually ESP32 controller | 1 |
| destination_device_id | INTEGER FK devices | No | Usually Raspberry Pi | 2 |
| boot_id | TEXT NOT NULL | No | Sender startup-session token | B-20261022-01 |
| sequence_no | INTEGER NOT NULL | No | Monotonic within boot session | 308 |
| message_type | TEXT CHECK | No | METAL_SAMPLE, DETECTION_EVENT, HEARTBEAT, ERROR, ACK | DETECTION_EVENT |
| payload_json | TEXT NOT NULL | No | Validated JSON frame with declared protocol schema | {"zone_id":2,"signal":142.5} |
| received_at | TEXT NOT NULL | No | Pi receive time UTC | 2026-10-22T09:20:01Z |
| parse_status | TEXT CHECK | No | ACCEPTED, DUPLICATE, INVALID, REJECTED | ACCEPTED |

Integrity / processing rules: UNIQUE(source_device_id,boot_id,sequence_no,message_type). Sender payload must not be trusted without schema and range validation.

## Table DD-13. dashboard_sessions

Authentication and authorized access tracking without storing plaintext session tokens.

| Field | SQLite type / key | Null? | Definition / validation | Example |
| --- | --- | --- | --- | --- |
| session_id | TEXT PK | No | Opaque generated session identifier/hash | s_… |
| user_id | INTEGER FK users | No | Associated account | 1 |
| created_at | TEXT NOT NULL | No | Session start UTC | 2026-10-22T08:05:00Z |
| expires_at | TEXT NOT NULL | No | Expiry UTC | 2026-10-22T16:05:00Z |
| revoked_at | TEXT | Yes | Session revocation UTC | NULL |
| client_label | TEXT | Yes | Coarse browser/client descriptor | Guard workstation |

Integrity / processing rules: Prefer framework-provided secure server-side session handling. Never log secret cookie values.

## Table DD-14. system_event_logs

General warning, maintenance and application state records not represented as physical screening events.

| Field | SQLite type / key | Null? | Definition / validation | Example |
| --- | --- | --- | --- | --- |
| event_id | INTEGER PK | No | Event identifier | 9001 |
| device_id | INTEGER FK devices | Yes | Originating device | 2 |
| event_category | TEXT CHECK | No | BOOT, SHUTDOWN, ERROR, CONFIG, MAINTENANCE, NETWORK | NETWORK |
| event_level | TEXT CHECK | No | INFO, WARNING, ERROR | WARNING |
| event_message | TEXT NOT NULL | No | Operational message | ESP32 UART timeout |
| event_time | TEXT NOT NULL | No | UTC occurrence timestamp | 2026-10-22T09:24:00Z |
| details_json | TEXT | Yes | Optional structured diagnostics | {"attempt":2} |

Integrity / processing rules: Retention and log rotation required on microSD to avoid uncontrolled growth.

# 3. ESP32-to-Raspberry Pi structured message data dictionary

The document’s ESP32 circuit uses GPIO25 for a coil-control waveform, but it does not establish a complete final metal receive front-end or a production protocol. The following frame defines the proposed six-zone integration contract. It is not represented as a confirmed installed interface.

| Field | Type | Constraint / meaning | Illustrative value |
| --- | --- | --- | --- |
| protocol_version | string | Required; API/firmware compatibility | 1.0 |
| device_code | string | Required; registered ESP32 | ESP32-01 |
| boot_id | string | Required; unique per power cycle | B-20261022-01 |
| sequence_no | integer | Required; monotonic within boot session | 308 |
| event_type | enum | METAL_SAMPLE / DETECTION_EVENT / HEARTBEAT / ERROR | DETECTION_EVENT |
| zone_id | integer | 1 through 6 for metal events | 4 |
| zone_code | enum | Matches six-row master data exactly | R_UPPER |
| signal_value | number | Nonnegative/within calibrated sensor range | 142.50 |
| signal_unit | enum | Must match device calibration configuration | ADC_COUNT |
| threshold_value | number | Logged threshold from channel | 120.00 |
| metal_detected | boolean | Determined by approved channel algorithm | true |
| event_time | ISO-8601 timestamp | ESP32 measurement time only if synchronized; else Pi receive time canonical | 2026-10-22T09:20:01Z |
| quality_flag | enum | VALID / NOISY / SATURATED / ERROR | VALID |
| checksum_or_crc | hex/string | Optional link-frame integrity validation | A1B2 |

Processing contract: ESP32 samples coils in a validated acquisition schedule → frame validation at Raspberry Pi → map zone_id to metal_zones → deduplicate by device/boot/sequence → insert controller_messages and metal_detection_logs in one controlled transaction → evaluate alert rules → store alerts → broadcast permitted event data to authenticated Flask-SocketIO clients. Offline or ambiguous records remain auditable.

# 4. Dashboard and data transfer dictionary

| Output / event | Direction | Payload elements | Source of truth / consumer |
| --- | --- | --- | --- |
| live_status | Pi → dashboard | gate_id, device_status, timestamp, active_zones, temperature, occupancy | device_heartbeats, metal_zones, latest logs |
| temperature_event | Pi → dashboard | temp_id, passage_id, measured_c, threshold_c, result, time | temperature_logs |
| metal_event | Pi → dashboard | detection_id, passage_id, zone_id, zone_code, signal, flag, time | metal_detection_logs JOIN metal_zones |
| headcount_event | Pi → dashboard | count_id, direction, occupancy, time | headcount_logs |
| alert_created | Pi → dashboard | alert_id, type, severity, zone_id, message, status, time | alerts |
| alert_acknowledge | dashboard → Pi | alert_id, user session, action time | Authenticated update to alerts + audit |
| setting_update | dashboard → Pi | setting_name, new_value, request identity, reason | system_settings + settings_audit_logs |
| historical_query | dashboard → Pi | from_time, to_time, types, zone filters, pagination | Server-side filtered query; authorized access |

Transport: Local browser communicates with Raspberry Pi Flask APIs and Flask-SocketIO over campus LAN. Require authentication, CSRF protection for state-changing requests, access logs, and transport security suitable for site deployment. The ESP32 is not the system’s database or browser server.

# 5. Relationships, constraints, and indexing

| Relationship | Cardinality | Rule |
| --- | --- | --- |
| devices → metal_zones | 1 : 0..N | Each zone may have a registered coil device; reassignment audited |
| passages → headcount_logs | 1 : 0..1 | Count only confirmed transit |
| passages → temperature_logs | 1 : 0..N | Readings can be retried |
| passages → metal_detection_logs | 1 : 0..N | Multiple zones can fire during same transit |
| metal_zones → metal_detection_logs | 1 : 0..N | Every metal record belongs to one of six positions |
| metal_detection_logs → alerts | 1 : 0..N | Metal alert retains source FK |
| temperature_logs → alerts | 1 : 0..N | Thermal alert retains source FK |
| users → alerts | 1 : 0..N | Ack actor, nullable until acknowledged |
| users → system_settings | 1 : 0..N | Administrative setting editor |
| system_settings → settings_audit_logs | 1 : 0..N | Revision trail |

SQLite setup: PRAGMA foreign_keys=ON for each connection; CHECK constraints for enums and numeric ranges; index all foreign-key columns and time-based event fields (headcount_logs.event_time, temperature_logs.reading_time, metal_detection_logs.detection_time, alerts.alert_time), plus composite index (metal_detection_logs.zone_id, detection_time). Use UTC ISO-8601 TEXT consistently; SQLite does not enforce traditional VARCHAR or DATETIME semantics.

Alert source validation: METAL alerts require a metal_detection_id; TEMPERATURE alerts require a temperature_log_id. Operational fault alerts may reference a device/event record instead. A direct FK to two unrelated tables through one generic source_id is invalid relational design and is therefore removed.

# 6. Required modifications to the manuscript and prototypes

Update Chapter 1 scope, limitations, objectives and definition of terms to explicitly state six physically distinct left/right vertical metal zones; remove or separately scope the BAG designation.

Update the mechanical layout and labels to Z01–Z06, matching the left and right upper/middle/lower detector panels. Show the six physical coil channels and their unique wires/addresses.

Revise the previous four-channel ESP32 wiring diagram: six independently addressable channels and a complete tested sensing front-end are required; the earlier GPIO25/26/27/14 mapping covers only four channels.

Update all DFDs, system architecture, dashboard mockups, testing forms, zone confusion matrices, alert messages, ERD and SQL schema to the same six-zone contract.

Before construction freeze, choose the actual metal sensing method and module, confirm whether the six channels operate independently or multiplexed, document feedback/readout pins and validate electrical safety.

Populate the six-zone master table and assign exact production GPIO pins only after hardware interface verification. The data dictionary specifies data responsibilities, not unverified wiring.

# 7. Validation and acceptance evidence

| Verification | Dataset / expected artifact | Acceptance interpretation |
| --- | --- | --- |
| Zone integrity | Six row metal_zones table and six-coil mapping sheet | Each physical channel reliably produces the correct zone_id |
| Signal correctness | Samples with signal units, thresholds and calibration | Measured sensitivity / false alarms per zone; no invented performance |
| Passage linkage | Test traces correlating beams, temperature and zone detections | No accidental merging of adjacent passages |
| Headcount | Labeled ENTRY/EXIT test sequences | Direction counts reconcile against observers |
| Persistence | Repeated insert, power loss and restart tests | No database corruption or duplicate event inflation |
| Communication | ESP32 UART trace, sequence IDs, recovery tests | Frames parsed, deduplicated and fault states visible |
| Dashboard | Time-stamped screen recording / logs | Updates match database and access is restricted |
| Privacy | Access policy and data retention review | No unnecessary personal identification stored |

Results remain pending until actual hardware installation, trials, and reference comparisons are completed. The inclusion of explicit validation columns supports a scientifically defensible prototype chapter without fabricating empirical outcomes.